Assignment Module 7 : Network fundamental

1- Which of the following messages in the DHCP process are broadcasted?

(Choose two)

A. Request

B. Offer

C. Discover

D. Acknowledge

Answer:-A. Request C. Discover

2- Which command would you use to ensure that an ACL does not block web-based TCP traffic?

A. permit any

B. permit tcp any any eq 80

C. permit tcp any eq 80

D. permit any any eq tcp

Answer:B. permit tcp any any eq 80

3-Explain Network Topologies?

Answer:

Network topology refers to the physical or logical layout of a network. Common types :

- Bus Topology: All devices share a single cable.

- Star Topology: All devices connect to a central switch or hub.

- Ring Topology: Devices are connected in a circular manner.

- Mesh Topology: Devices are interconnected with multiple paths for redundancy.

- Hybrid Topology: Combination of two or more topologies.

4-Explain TCP/IP Networking Model?

Answer:

The TCP/IP model of four layers:

- Application Layer: Provides network services (HTTP, FTP, SMTP).

- Transport Layer: Ensures data delivery (TCP, UDP).

- Internet Layer: Handles logical addressing and routing (IP, ICMP).

- Network Access Layer: Deals with physical transmission (Ethernet, Wi-Fi).

5-Explain LAN and WAN Network?

Answer:

- LAN (Local Area Network):

  - Covers a small area (home, office, school).

  - High speed and low cost.

- WAN (Wide Area Network):

  - Covers a large geographical area (cities, countries).

  - Uses leased lines, fiber, or satellite.

  - Example: The Internet.

6-Explain Operation of Switch ?

Answer:

A switch operates at the Data Link Layer (Layer 2). It:

- Learns MAC addresses from incoming frames.

- Stores MAC addresses in a MAC table.

- Forwards data only to the destination port.

- Reduces network collisions and improves performance.

7-Describe the purpose and functions of various network devices Make list of the appropriate media, cables, ports, and connectors to connect switches to other.?

Answer:

Network Devices & Functions:

- Switch: Connects devices within a LAN.

- Router: Connects different networks and routes data.

- Hub: Broadcasts data to all ports.

- Modem: Connects LAN to ISP.

- Access Point: Provides wireless connectivity.

Media, Cables, Ports & Connectors:

- Media: Copper, Fiber optic, Wireless

- Cables: UTP (Cat5e, Cat6) ,Fiber optic cable

- Ports: Ethernet ports (Fast Ethernet, Gigabit)

- Connectors: RJ-45 (copper)

8-Define Network devices and hosts ?

Answer:

Network Devices: Hardware used to connect and control network traffic (switches, routers, firewalls).

Hosts: End devices that send or receive data on a network (computers, servers, printers, smartphones).