Assignment Module 5

Network Fundamentals and Building Networks

Section 1: Multiple Choice

1. What is the primary function of a router in a computer network?

a) Assigning IP addresses to devices

b) Providing wireless connectivity to devices

c) Forwarding data packets between networks

d) Managing user authentication and access control

Answer:- c) Forwarding data packets between networks

2.What is the purpose of DHCP (Dynamic Host Configuration Protocol) in a computer network?

a) Assigning static IP addresses to devices

b) Resolving domain names to IP addresses

c) Managing network traffic and congestion

d) Dynamically assigning IP addresses to devices

Answer:- d) Dynamically assigning IP addresses to devices

3.Which network device operates at Layer 2 (Data Link Layer) of the OSI model and forwards data packets based on MAC addresses?

a) Router

b) Switch

c) Hub

d) Repeater

Answer:- b) Switch

4.Which network topology connects all devices in a linear fashion, with each device connected to a central cable or backbone?

a) Star

b) Bus

c) Ring

d) Mesh

Answer:- b) Bus

Section 2: True or False

5. True or False: A VLAN (Virtual Local Area Network) allows network administrators to logically segment a single physical network into multiple virtual networks, each with its own broadcast domain.

Answer:-True

6. True or False:TCP (Transmission Control Protocol) is a connectionless protocol that provides reliable, ordered, and error-checked delivery of data packets over a network.

Answer:-False

TCP is connection-oriented, not connectionless.

It ensures reliable, ordered, and error-checked data delivery.

(The description matches TCP, but "connectionless" applies to UDP( User Datagram Protocol) not TCP.)

7. True or False:A firewall is a hardware or software-based security system that monitors and controls incoming and outgoing network traffic based on predetermined security rules.

Answer:-True

Section 3: Short Answer

8. Describe the steps involved in setting up a wireless network for a small office or home office (SOHO) environment.

Answer:-

### Steps to Set Up a Wireless Network for a SOHO

- Plan the Network: Decide how many devices will connect and the internet speed needed.

- Install Equipment: Get an internet connection and a wireless router.

- Connect Hardware: Connect the router to the modem and power it on.

- Configure Settings: Set the SSID (network name), strong Wi-Fi password, and enable security (WPA2/WPA3).

- Test and Secure: Connect devices, check signal strength, and update router firmware regularly.

In short: Plan, connect, configure, test, and secure the wireless network.

9.Demonstrate how to configure a router for Internet access using DHCP (Dynamic Host Configuration Protocol).

Answer:-

To configure a router to access the Internet using DHCP (Dynamic Host Configuration Protocol).

### Equipment Required:

- Wireless/Wired Router

- Modem or Broadband Connection from ISP

- Computer or Laptop

- Ethernet Cables

### Theory:

DHCP (Dynamic Host Configuration Protocol) is used to automatically assign IP addresses to network devices.
When a router is configured to use DHCP on its WAN interface, it automatically obtains an IP address, subnet mask, default gateway, and DNS server from the ISP, eliminating the need for manual configuration.

- Connect the Devices:

  - Connect the WAN/Internet port of the router to the modem using an Ethernet cable.

  - Connect your computer to one of the router’s LAN ports (or via Wi-Fi).

  - Power on the modem and router.

- Access Router Configuration Page:

  - Open a web browser and type the router’s IP address (e.g., 192.168.0.1 or 192.168.1.1).

  - Log in using the default username and password (usually admin / admin).

- Configure WAN (Internet) Settings:

  - Go to Network Settings → WAN Setup → Internet Setup.

  - Select Connection Type: DHCP (Dynamic IP) or Obtain IP Automatically.

  - Save the settings.

- Verify Connection:

  - Open Status or WAN Information page.

  - Check if the router has received an IP address, gateway, and DNS from the ISP.

- Test Internet Access:

  - Open a web browser and visit any website (e.g., https://www.google.com).

  - If the page loads successfully, the configuration is complete.

DHCP configuration commands :-

Router> enable

Router# configure terminal

Router(config)# service dhcp

Router(config)# ip dhcp pool DRASHTI

Router(dhcp-config)# network 192.168.1.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.1.1

Router(dhcp-config)# dns-server 192.168.1.1

Router(dhcp-config)# exit

Section 4: Practical Application

10.Discuss the importance of network documentation in the context of building and managing network.

Answer:-

## Network Documentation

### Introduction

Network documentation is a very important part of managing a computer network. It means keeping a proper record of all the information about the network, such as devices, connections, configurations, and settings. In simple terms, it is like keeping a map or a report of how the network is built and how it works. This helps network administrators to understand, maintain, and troubleshoot the network easily whenever needed.

### Definition

Network documentation is the process of recording and maintaining detailed information about a computer network’s structure, components, and settings. It includes diagrams, IP addressing schemes, device configurations, cabling details, and security settings. In other words, it provides a complete picture of how a network is designed and operated.

### Purpose of Network Documentation

The main purpose of network documentation is to make the network easy to manage, maintain, and repair. When all network information is properly recorded, it becomes easier to plan upgrades, fix problems, and ensure that everything is working correctly. Documentation also helps new staff members understand the network quickly.

### Key Components of Network Documentation

- Network Topology Diagram:
 This is a diagram that shows how all the devices such as routers, switches, and computers are connected to each other. It helps understand the structure of the network.

- IP Addressing Scheme:
It contains details of all the IP addresses and subnets used in the network. This helps avoid address conflicts and makes troubleshooting easier.By keeping a proper record of IP addresses and using tools like Ping and Traceroute, network administrators can easily monitor, manage, and troubleshoot the network efficiently.

- Device Configuration Files:
 These are the settings and commands used to configure devices like routers and switches. Keeping a backup of these configurations helps in quick recovery if a device fails.

- Hardware and Software Inventory:
 This includes a list of all network devices (like routers, switches, and servers) and software used in the network, along with their versions and serial numbers.

- Cabling Details:
 Information about the types of cables, port numbers, and how devices are physically connected. It helps in maintenance and repairs.

- Security Policies:
 This includes rules related to network security such as firewall settings, access controls, and password policies. It helps protect the network from unauthorized access.

- Backup and Recovery Plans:
 These are the steps and procedures to recover the network if it fails or gets damaged due to any reason like hardware failure or cyberattacks.

### Conclusion

In conclusion, network documentation is a very important part of network management. It helps in designing, maintaining, troubleshooting, and securing the network. Without proper documentation, it becomes very difficult to solve network problems or make future upgrades. Therefore, every organization should maintain clear, complete, and updated network documentation for smooth and efficient network operation.