Assignment Module 8

Routing and Switching

1. 	Explain Switch ?

Answer:-A switch is a networking device used to connect multiple computers or devices within a LAN (Local Area Network).

It receives data packets and forwards them only to the specific device (port) that needs the data. This makes communication faster and more secure compared to a hub.

Switches operate at Layer 2 (Data Link Layer) of the OSI model and use MAC addresses to forward data.

2.Explain Switch Boot Sequence?

Answer:-

When a Cisco switch starts, it follows a specific boot sequence:

- POST (Power-On Self Test):
 Checks hardware components like CPU, memory, and ports.

- Load the Boot Loader:
 A small program stored in ROM is loaded.

- Find and Load IOS:
 Boot loader locates the Cisco IOS software in flash memory and loads it.

- Load Startup Configuration:
 The switch loads the configuration file from NVRAM.

3.Explain Three Methods to access Switch Command Line Interface?

Answer:-

Console Access:

- Physical connection using a console cable (RJ-45 or USB).

- Used for initial configuration.

Telnet Access:

- Remote access over the network.

- Transmits data as plain text (not secure).

### Auxiliary (AUX) Port Access

- AUX port is similar to the console port but is used through a modem.

- Allows remote out-of-band management when network access is not available.

- Access is still through terminal software but via a modem connection.

4.Explain and Configure the Cisco Internet Operating System?

Answer:-

Cisco IOS (Internetwork Operating System) is the software used in Cisco routers and switches. It controls all routing, switching, security, and network operations.

Features of Cisco IOS:

- Command-Line Interface (CLI)

- Supports routing protocols (OSPF, EIGRP, RIP)

- Supports switching features (VLANs,trunking)

### Basic IOS Configuration

Router> enable // privilege Mode

Router# configure terminal //global configuration mode

R1(config)#interface FastEthernet0/0

//Interface Confirmation Mode

R1(config-if)# ip address 192.168.1.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

5.Explain Switch Port?

Answer:- A switch port is a physical interface on a switch where you connect devices such as PCs, printers, routers, or other switches.

### Types of Switch Ports

- Access Port

  - Assigned to one VLAN

  - Used for end-user devices

#switchport mode access

#switchport access vlan 10

- Trunk Port

  - Carries multiple VLANs

  - Used between switches or switch–router links

#switchport mode trunk

#switchport trunk allowed vlan 10,20,30

4-R1,R2,R3,andR4 havetheir Fast Ethernet 0/0interfacesattached tothe same VLAN.A networkengineerhastypedaconfigurationforeachrouterbyusingawordprocessor.

Hewilllatercopyandpastetheconfigurationintotherouters.Examinethefollowing exhibit,whichlistsconfigurationforthefourrouters,astypedbythenetworkengineer. Assumingthatallfourrouterscanpingeachother’sLANIPaddressesafterthe configurationhasbeenapplied,choosetheroutersthatwillbeabletoformaneighbor relationshipwiththeotherroutersontheLAN.(Youcanassumethat,ifnotshowninthe exhibit,allotherrelatedparametersarestillsettotheirdefaults.)(Choosetwo)

A.  R1

B.  R2

C.  R3

D.  R4

E. Noneoftherouterswillexchangerouting information.

Answer:- A.R1 and B.R2

3-  enable secret                                                                           	[password]                                                                                    is hashedusingthe            	algorithm.

A.	MD5

B.   AH

C.   PSK

D.    ESP

E.   WPA2

Answer:-

4-   An engineer connects to Router R1 and issues as how ip ospf neighbor command. The status of neighbor 2.2.2.2 lists FULL/BDR. What does the BDR mean?

A.	R1 is an Area Border Router.

B.   R1 is a back up designated router.

C.   Router 2.2.2.2 is an Area Border Router.

D.    Router 2.2.2.2 is a back up designated router.

Answer:-D.    Router 2.2.2.2 is a back up designated router.

5-   Which comman  disused to view the neighbor discovery table on a PC?

A.	show ipv6 neighbor

B.   show ipv6 neighbors

C.   netsh interface ipv6 show neighbor

D.    netsh interface ipv6 show neighbors

Answer:-C.   netsh interface ipv6 show neighbor

6-   What type of variable is being shown? Routers=[R1,R2,R3]

A.List

B.   Dictionary

C.   Simple

D.    Unsignedintegers

Answer:-A.List

7-   Identify the fields in an IPv4 header.(Choosethree)

A.	Host component

B.   Time to Live

C.   Source address

D.    Destination address

E.  Network

Answer:-

### B. Time to Live

### C. Source address

### D. Destination address